NAME :  GNANAHASAN M

REG NO: (192512347)

COURSE NAME : ANALOG AND DIGITAL COMMUNICATION (ECA0802)

| EX NO: 8b | SIMULATION AND ANALYSIS OF QPSK MODULATION USING CONSTELLATION DIAGRAMS IN MATLAB |
| --- | --- |
| DATE: |  |

Objective

i) To generate QPSK symbols from a binary data sequence using quadrature phase-shift keying principles.

ii) To plot and analyze the QPSK signal constellation in the In-Phase (I) and Quadrature (Q) plane.

iii) To investigate the effect of noise on constellation points and evaluate the resulting degradation in symbol detection performance.

Matlab code

clc;

clear;

close all;

%% Objective 1: Generate QPSK Symbols

N = 1000;                      	% Number of bits

data = randi([0 1], N, 1);     	% Random binary sequence

% Ensure even number of bits

if mod(N,2)~=0

data = [data; 0];

N = N + 1;

end

% Group bits into pairs

bit_pairs = reshape(data,2,[])';

% QPSK Mapping

qpsk_symbols = zeros(size(bit_pairs,1),1);

for k = 1:size(bit_pairs,1)

if isequal(bit_pairs(k,:),[0 0])

qpsk_symbols(k) = (1+1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[0 1])

qpsk_symbols(k) = (-1+1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[1 1])

qpsk_symbols(k) = (-1-1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[1 0])

qpsk_symbols(k) = (1-1j)/sqrt(2);

end

end

%% Objective 2: Plot QPSK Constellation

scatter(real(qpsk_symbols),imag(qpsk_symbols), 30);

xlim([-1 1])

ylim([-1 1])

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title('Ideal QPSK Constellation');

%% Objective 3: Add Noise and Evaluate Performance

SNR = 5;

rx_signal = awgn(qpsk_symbols,SNR,'measured');

figure;

scatter(real(rx_signal),imag(rx_signal), 15);

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title(['Noisy QPSK Constellation (SNR = ',num2str(SNR),' dB)']);

%% Symbol Detection

detected_bits = zeros(length(rx_signal)*2,1);

for k = 1:length(rx_signal)

I = real(rx_signal(k));

Q = imag(rx_signal(k));

if I>0 && Q>0

bits = [0 0];

elseif I<0 && Q>0

bits = [0 1];

elseif I<0 && Q<0

bits = [1 1];

else

bits = [1 0];

end

detected_bits(2*k-1:2*k) = bits;

end

%% BER Calculation

num_errors = sum(data ~= detected_bits);

BER = num_errors/N;

fprintf('Number of Transmitted Bits : %d\n',N);

fprintf('Number of Bit Errors   	: %d\n',num_errors);

fprintf('Bit Error Rate (BER)   	: %f\n',BER);

PRODUCER

- Initialize the MATLAB environment and define the number of bits.

- Generate a random binary sequence consisting of 0s and 1s.

- Group the binary sequence into pairs of bits.

- Apply QPSK modulation by mapping each bit pair to a unique constellation point.

- Generate the corresponding complex QPSK symbols.

- Verify that four distinct symbol values are obtained.

- Store the generated symbols for constellation analysis.

OUTPUT GRAPH

Result

QPSK symbols were successfully generated from the binary data sequence using quadrature phase-shift keying modulation. The QPSK constellation was successfully plotted in the In-Phase (I) and Quadrature (Q) plane, showing four distinct symbol points corresponding to the bit pairs 00, 01, 11, and 10.