NAME :  GNANAHASAN M

REG NO: (192512347)

COURSE NAME : ANALOG AND DIGITAL COMMUNICATION (ECA0802)

| EX NO: 9 | SIMULATION OF CONVOLUTIONAL CODES AND ANALYSIS OF ENCODED OUTPUT BIT SEQUENCES USING MATLAB |
| --- | --- |
| DATE: |  |

Objectives

(i) To implement convolutional encoding for a given binary input sequence using a rate  convolutional encoder and tabulate the corresponding encoded output pairs.

(ii) To visualize and compare the input bit sequence and encoded output bit sequence using MATLAB plots to analyze the redundancy introduced by convolutional coding.

MATLAB code

clc;

clear;

close all;

% Fixed input sequence

input_bits = [1 0  0 1 1];

% Convolutional encoder

trellis = poly2trellis(3,[7 5]);

encoded_bits = convenc(input_bits,trellis);

% Separate encoded bits into pairs

encoded_pairs = reshape(encoded_bits,2,[])';

% Create output strings

for i = 1:length(input_bits)

output_pair{i} = sprintf('%d%d', ...

encoded_pairs(i,1),encoded_pairs(i,2));

end

% Create table

Bit_Position = (1:length(input_bits))';

Input_Bit = input_bits';

Output_Encoded = output_pair';

T = table(Bit_Position,Input_Bit,Output_Encoded);

disp('Convolutional Encoding Table');

disp(T);

% Plot

figure;

subplot(2,1,1)

stairs(input_bits,'LineWidth',1.5)

ylim([-0.2 1.2])

title('Input Bit Sequence')

xlabel('Bit Position')

ylabel('Bit Value')

grid on

subplot(2,1,2)

stairs(encoded_bits,'LineWidth',1.5)

ylim([-0.2 1.2])

title('Encoded Output Bit Sequence')

xlabel('Bit Position')

ylabel('Bit Value')

Procedure

- Open MATLAB and create a new script file.

- Initialize the MATLAB workspace using clc, clear, and close all.

- Define the input bit sequence as .

- Create a convolutional encoder trellis using a constraint length of 3 and generator polynomials .

- Encode the input sequence using the convenc() function.

- Group the encoded bits into pairs corresponding to each input bit.

- Create and display a table containing the bit position, input bit, and encoded output pair.

- Observe the encoded output pairs generated by the encoder.

grid on

Objective ::

To implement convolutional encoding for a given binary input sequence using a rate  convolutional encoder and tabulate the corresponding encoded output pairs.

Procedure

- Open MATLAB and create a new script file.

- Initialize the MATLAB workspace using clc, clear, and close all.

- Define the input bit sequence as .

- Create a convolutional encoder trellis using a constraint length of 3 and generator polynomials .

- Encode the input sequence using the convenc() function.

- Group the encoded bits into pairs corresponding to each input bit.

- Create and display a table containing the bit position, input bit, and encoded output pair.

- Observe the encoded output pairs generated by the encoder.

Tabulation

| Bit position | Input bit | Encoded sequence |
| --- | --- | --- |
| 1 | 1 | 11 |
| 2 | 0 | 10 |
| 3 | 0 | 11 |
| 4 | 1 | 11 |
| 5 | 1 | 01 |

OUTPUT

Result

The convolutional encoder was successfully implemented using a constraint length of 3 and generator polynomials . The input bit sequence was encoded into the output sequence 1110111101, and the corresponding encoded output pairs were tabulated successfully.